IoT Temperature Alert System

Project report: ESP32, Firebase and Node-RED

1. Objective

The goal of this project is to monitor temperature over the internet and get an alert when it gets too high. An ESP32 reads temperature and humidity from a DHT11 sensor and sends the values to Firebase. Node-RED picks up the data from Firebase, shows it on a dashboard and gives an alert if the temperature goes above the limit.

2. Components Used

- ESP32 Dev Module (has Wi-Fi built in)

- DHT11 temperature and humidity sensor

- Firebase Realtime Database

- Node-RED with node-red-dashboard

- Arduino IDE with the Adafruit DHT sensor library and Adafruit Unified Sensor library

3. How It Works

The sensor sends data to the ESP32, the ESP32 sends it to Firebase, and Node-RED reads it from Firebase and shows it on the dashboard with an alert.

- The ESP32 reads the DHT11 every 5 seconds.

- It connects to Wi-Fi and sends the reading to Firebase over HTTPS.

- Node-RED asks Firebase for the latest reading every 5 seconds.

- The temperature is shown on a gauge, and a switch node checks it against the limit to decide if an alert is needed.

4. Circuit Connections

- DHT11 VCC to ESP32 3.3V

- DHT11 DATA to ESP32 (D4)

- DHT11 GND to ESP32 GND

If a bare 4-pin DHT11 is used instead of the 3-pin module, a 10 kΩ resistor is needed between DATA and VCC.

5. Firebase

The ESP32 sends the reading to the path /temperature using an HTTP PUT request every 5 seconds. Each new reading replaces the old one. It is stored like this:

{ "temperature": 31.4, "humidity": 84, "timestamp": 1791823969984 }

The timestamp is in milliseconds and comes from an NTP server. Node-RED reads the same path with an HTTP GET request.

6. Node-RED Flow

The flow file is flows.json in the project files. It has these nodes:

- inject: starts a fetch every 5 seconds

- http request: gets the data from the Firebase URL

- function: takes the temperature value and converts it to a number for the gauge

- ui_gauge: shows the temperature on the dashboard (0 to 50 °C)

- switch: checks the temperature against the limit

- set msg.payload: writes the alert message

- debug: shows the readings and alerts so I can check them

7. Dashboard

The dashboard is called IoT Temperature Monitor. It has one card, Sensor readings, with a Temperature (°C) gauge. The gauge changes each time a new reading comes in from Firebase.

8. Alert

The alert is handled by the switch node. On every fetch the temperature is compared with 31 °C. If it is above 31 °C the message "high temperature detected" is shown. If it is 31 °C or below there is no alert. For example, 31.4 °C gives an alert and 30.4 °C does not.

9. Testing

I did four tests and all of them passed.

- ESP32 powered on with Wi-Fi: new values showed up in Firebase every 5 seconds.

- Node-RED fetch: the debug node showed temperature, humidity and timestamp.

- Temperature 30.4 °C: the gauge showed 30.4 and there was no alert.

- Temperature 31.4 °C: the gauge showed 31.4 and the alert appeared.

Problem I ran into: the gauge showed 0 even though the debug nodes showed the right values. The data was reaching the gauge as a whole object, so the gauge could not read it. I fixed it by adding a function node that takes msg.payload.temperature and converts it to a number before the gauge.

10. Limitations

- Node-RED checks Firebase every 5 seconds, so the alert is not instant.

- Only the latest reading is saved, so there is no history or graph.

- Alerts only show in the Node-RED debug panel. There is no email, SMS or notification.

- Only a high temperature limit is set. A low limit can be added as another switch rule.

- I did not test Wi-Fi loss, sensor disconnection or a low temperature alert.

- The DHT11 is only accurate to about ±2 °C and works from 0 to 50 °C.

- The Firebase rules are open for testing, which is not safe for real use.

- If Wi-Fi goes down, readings are lost because the ESP32 does not store them.

11. Conclusion

The system reads the temperature from the DHT11, stores it in Firebase, shows it on a Node-RED dashboard and gives an alert when it goes above 31 °C. I tested the whole path from sensor to cloud to dashboard and it works. It can be improved with a low temperature limit, email or Telegram alerts, saving history and securing Firebase.

12. Project Files

- IoT_TEMP.ino: ESP32 code

- flows.json: Node-RED flow

- circuit_diagram.png: circuit diagram

- screenshots/: Firebase database, Node-RED flow, dashboard and alert